RISC-V VAES Vector Cryptography Coprocessor
Microarchitecture Design and Functional Verification

| Project Deliverable | SystemVerilog RTL, self-checking Verilator testbench, design PDF, ISA markdown |
| --- | --- |
| Target Instructions | vaes.sbox.v, vaes.srow.v, vaes.mcol.v, vaes.ark.v |
| Execution Model | Normal 5-stage pipeline: IF / ID / EX / MEM / WB |
| Verification Environment | Verilator with testbench reference model, directed tests, randomized tests, and SVA |

Fig. 1. Top-level block diagram of the coprocessor and its verification boundaries.

# 1. Introduction

This design implements a dedicated AES-oriented vector coprocessor for RISC-V using four custom vector instructions. The datapath uses a normal five-stage pipeline and a 32-entry, 128-bit vector register file. The coprocessor communicates with the host through AXI4-Stream, where the input stream carries instruction and data packets and the output stream returns 128-bit results.

The verification environment is built for Verilator. A SystemVerilog class-based bit-accurate reference model is used only inside the testbench and scoreboard. It is not reused as any RTL pipeline stage. The regression includes a NIST known-answer test, directed variable-VL tests, randomized AES encryptions, randomized instruction streams, output backpressure, and SVA checks for both interface and internal control legality.

# 2. Design targets and assumptions

- Four required custom vector instructions are implemented exactly: vaes.sbox.v, vaes.srow.v, vaes.mcol.v, and vaes.ark.v.

- The vector register file contains 32 registers, each 128 bits wide.

- The pipeline is a normal five-stage IF / ID / EX / MEM / WB structure. Register read is folded into the ID stage; there is no separate RR stage.

- Dedicated parallel SubBytes and MixColumns units exist in EX, and both are modeled as fixed multi-cycle operations.

- The legal vector lengths are 4, 8, 12, and 16 bytes so that AES column granularity is preserved.

- The reference model remains strictly inside the Verilator testbench. It is used for sequence generation, state mirroring, and scoreboard comparisons only.

# 3. Interface definition

The coprocessor exposes one AXI4-Stream slave input and one AXI4-Stream master output. The slave stream carries either instruction packets or auxiliary control/data packets.

## 3.1 AXI4-Stream ingress

| Signal | Width | Description |
| --- | --- | --- |
| s_axis_tvalid | 1 | Input packet valid |
| s_axis_tready | 1 | Coprocessor ready to accept current packet |
| s_axis_tdata | 256 | Tagged instruction/data packet |
| s_axis_tlast | 1 | Always asserted for one-packet transactions |

## 3.2 AXI4-Stream egress

| Signal | Width | Description |
| --- | --- | --- |
| m_axis_tvalid | 1 | Result packet valid |
| m_axis_tready | 1 | Downstream sink ready |
| m_axis_tdata | 128 | Returned vector result |
| m_axis_tlast | 1 | Always asserted with each result packet |

## 3.3 Ingress packet format

| Bits | Field | Meaning | Used by |
| --- | --- | --- | --- |
| 255:248 | pkt_type | Packet kind | LOAD / INST / CFG / STORE |
| 247:216 | word0 | Raw 32-bit custom instruction word | INST |
| 215:211 | reg_idx | Vector register index | LOAD / STORE |
| 210:206 | vl | Vector length in bytes | CFG |
| 205:128 | reserved | Reserved and set to zero | All |
| 127:0 | payload | 128-bit vector data | LOAD |

# 4. Custom instruction set

All four custom instructions share the same 32-bit custom R-type format: funct6 | vm | vs2 | vs1 | funct3 | vd | opcode. The fixed fields are opcode = custom-0 (0001011), funct3 = 000, and vm = 1.

| Mnemonic | funct6 | Operands | Function | Latency |
| --- | --- | --- | --- | --- |
| vaes.sbox.v | 000000 | vd, vs1 | Apply AES S-box to each active byte | 2 cycles |
| vaes.srow.v | 000001 | vd, vs1 | Apply ShiftRows on active AES columns | 1 cycle |
| vaes.mcol.v | 000010 | vd, vs1 | Apply MixColumns on active AES columns | 2 cycles |
| vaes.ark.v | 000011 | vd, vs1, vs2 | XOR active bytes with round key | 1 cycle |

The full bit-level instruction definition is provided in the companion file `riscv_vaes_extension.md`. That file is the normative ISA specification for the deliverable.

# 5. Microarchitecture

The core is organized around a single instruction stream and a centralized vector register file. LOAD, CFG, and STORE packets are treated as control-plane operations on the AXI ingress. The four custom VAES instructions flow through the five-stage instruction pipeline.

Fig. 2. Five-stage pipeline and multi-cycle execution behavior.

## 5.1 Stage-by-stage behavior

• IF captures one raw 32-bit custom instruction from the ingress packet stream and snapshots the current VL.

• ID decodes the instruction, reads the vector sources, and applies scoreboard-based hazard checking.

• EX selects one of the four execution units. ShiftRows and AddRoundKey are single-cycle. SubBytes and MixColumns are modeled as fixed 2-cycle operations.

• MEM acts as a result holding stage so the five-stage structure remains explicit and normal.

• WB writes the destination register and clears the corresponding busy bit.

## 5.2 Execute-stage hardware

The execute stage instantiates four dedicated combinational units: `vaes_sbox_unit`, `vaes_srow_unit`, `vaes_mcol_unit`, and `vaes_ark_unit`. The SubBytes unit performs 16 parallel byte substitutions. The MixColumns unit computes all active AES columns in parallel. A lightweight control FSM keeps SubBytes and MixColumns in EX for two cycles before forwarding the result to MEM.

## 5.3 Hazard handling

A 32-bit busy-bit scoreboard tracks unresolved register destinations. The ID stage stalls when a source register is still busy or when a younger instruction would overwrite a destination that is already pending. This conservatively handles RAW and WAW hazards. Because the regression is correctness-driven rather than throughput-driven, no bypass network is used.

## 5.4 AXI handshake control

The ingress logic accepts instruction packets only when the IF stage is empty. LOAD/CFG/STORE packets are accepted only when the pipeline is empty and no egress result is pending. The result channel holds `m_axis_tvalid` until `m_axis_tready` is observed. SVA checks ensure the output payload remains stable while stalled.

# 6. Key RTL modules

| Module | Responsibility |
| --- | --- |
| vaes_common_pkg.sv | Shared constants, enums, instruction encoding/decoding helpers, and packet constructors. |
| vaes_aes_pkg.sv | Synthesizable AES helper functions used by the execution units. |
| vaes_regfile.sv | 32 x 128-bit vector register file with one write port and two asynchronous read ports. |
| vaes_sbox_unit.sv | Parallel SubBytes datapath. |
| vaes_srow_unit.sv | ShiftRows datapath. |
| vaes_mcol_unit.sv | Parallel MixColumns datapath. |
| vaes_ark_unit.sv | AddRoundKey datapath. |
| vaes_coproc_top.sv | Top-level pipeline, AXI handshake logic, scoreboard, and control FSM. |

# 7. Verification architecture

Fig. 3. Verification flow used by the Verilator testbench.

The testbench is pure SystemVerilog and targets Verilator. The stimulus source is a text sequence file that is parsed by the sequencer. Each parsed command is translated into an AXI4-Stream packet for the DUT. At the same time, the reference model mirrors the architectural state and predicts expected STORE results, which are queued for scoreboard comparison.

## 7.1 Reference model usage

The `vaes_ref_model` class provides a bit-accurate AES-128 model, key expansion, and architectural-state mirroring for the four custom instructions. It is never instantiated inside the RTL and it is not used as a pipeline stage. This separation keeps the implementation independent while still providing strong self-checking capability.

## 7.2 Directed tests

• NIST AES-128 known-answer test using plaintext `00112233445566778899aabbccddeeff` and key `000102030405060708090a0b0c0d0e0f`.

• Directed variable-VL tests at 4, 8, 12, and 16 bytes to exercise partial-width execution semantics.

• Hazard-heavy instruction streams that repeatedly overwrite the same state register.

## 7.3 Randomized tests

To remain Verilator-friendly out of the box, the randomized test generator uses solver-free constrained generation rather than relying on external SMT-based `randomize()` support. The generator writes random text sequence files and then re-reads them through the exact same sequencer path. Two random families are included:

• Random AES tests: random plaintext/key pairs, full-length VL = 16, full AES-128 round schedule.

• Random ISA tests: random VL chosen from {4, 8, 12, 16}, random LOADs, random instruction mixes, random STORE points, and randomized output backpressure.

## 7.4 Assertions

• Ingress packets must assert `TLAST` when accepted.

• The output channel must hold `TVALID`, `TDATA`, and `TLAST` stable while stalled.

• Control-state assertions tie `CTRL_EX_SBOX`, `CTRL_EX_MCOL`, and `CTRL_EGRESS` to legal internal conditions.

• Write-back is asserted only for destinations that are marked busy by the scoreboard.

# 8. Regression result used for sign-off

The package was compiled and executed with Verilator 5.38 in the preparation environment used for this deliverable. The shipped regression completed successfully with:

• 22 tests completed

• 65 output packets matched against the reference model

• Zero scoreboard mismatches

• Zero simulation runtime errors

# 9. How to run

The package includes a ready-to-use shell script and a Makefile target. The recommended command is `make run`. The script falls back to `python -m verilator` if the Verilator Python wrapper is used instead of a system `verilator` executable.

# 10. Delivered files

• `rtl/` — synthesizable SystemVerilog source

• `tb/` — self-checking SystemVerilog testbench, reference model, and example text sequences

• `riscv_vaes_extension.md` — ISA definition

• `docs/vaes_coproc_design.pdf` — this design specification

• `sim/run_verilator.sh` and `Makefile` — reproducible simulation entry points